Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Athabasca Valley

LAST UPDATED: February 18, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Station Photos	17

Tables and Figures

Figure 1 – Area topographic map showing AMS 07	8

Figure 2 – Aerial image showing AMS 07	9

Figure 3 – Plan view image for AMS 07 site	10

Figure 4 – Elevation view image for AMS 07 site	11

Figure 5 – Environment looking North	12

Figure 6 – Environment looking East	13

Figure 7 – Environment looking South	14

Figure 8 – Environment looking West	15

Figure 9 – Meteorological Tower	16

Figure 10 – Photo showing the inlet and sample manifold	17

Figure 11 – Curb shot of the monitoring station	18

Figure 12 –Photo of the front and the back of instrument rack	19

Figure 13 – Windrose (2021-2025)	20

# General Site Information

Revision Date: March 27, 2024

## Station

| Station ID | AMS 07 |
| --- | --- |
| Station name | Athabasca Valley |
| Date station established | 1977 |

## Location

| Station street address | Located on MacDonald Drive, Near the Athabasca River and McDonald Island |
| --- | --- |
| Legal land description | 9-20-089-09 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 56°44’00.21”N |
| Longitude | 111°23’25.80”W |
| UTM East | 476105.865 |
| UTM North | 6287773.834 |
| Nearest community | Fort McMurray |
| Community population | 75,186 |
| Census Year | 2021 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | N/A |
| Approval number | N/A |
| Contact Name | Wood Buffalo Environmental Association |
| Address | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Phone number | 780-799-4420 |
| Email address | info@wbea.org |

## Site Description

| Land use by sector | 0 – 90 degrees | Residential |
| --- | --- | --- |
|  | 91 – 180 degrees | Residential |
|  | 181 – 270 degrees | Residential |
|  | 271 – 360 degrees | River |
| Site elevation (m) (above sea level) | 497 m |  |
| Angle of elevation to nearby buildings | Greatest angle | 3 |
|  | Building direction | West |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | House |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10 m |
|  | Distance from station (m) | Attached to North end of monitoring station |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Wood Smoke | 10m | Wood Stove from shelter (due West) |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Asphalt road | Medium | 15 | Access road to MacDonald Island |
| Highway | High | 300 | Hwy 63 |
| Asphalt Road | Medium | 330 | City street |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Fort McMurray Water treatment Plant | Water treatment Plant | 1.22 | SW |
| McDonald Island | Sports Complex | 0.265 | NE |
| Eveready | Asphalt Plant | 3.71 | NW |
| Fort McMurray Waste Water Treatment Plant | Waste Water Treatment Plant | 3.84 | NW |
| Diversified | Main Bus Depo | 4.48 | NW |
| Suncor | Oil Sands Plant | 26.81 | N |
| Suncor | Tailings Pond | 15.81 | N |
| LaFarge | Concrete Plant | 3.06 | NW |
| Inland | Concrete Plant | 3.32 | NW |
| Burnco | Concrete Plant | 4.24 | NW |

# Station Equipment

Equipment Owner: Alberta EPA and WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Scientific | 43i-LTE | 1507864683 | 2020 | January, 1999 |
| TRS | Thermo Scientific | 43i-LTE | 1180540018 | 2019 | January, 1999 |
| TRS conv | CD-Nova | CDN-101 | 551 | 2019 | - |
| NOx/NO/NO2 | Thermo Scientific | 42i | 1160120024 | 2021 | January, 1999 |
| THC/CH4/NMHC | Thermo Scientific | 55i | 12300522720 | 2024 | January, 1999 |
| O3 | Thermo Scientific | 49i | 1507964700 | 2019 | January, 1999 |
| CO | Thermo Scientific | 48i-LTE | 1408761381 | 2016 | January, 1999 |
| PM2.5 | Teledyne | T640 | 645 | 2024 | January, 1999 |
| Time-Integrated |  |  |  |  |  |
| PM10 | Thermo Scientific | 2000i | 2000i 0361 1305 | 2016 | - |
| PM10 | Thermo Scientific | 2000i | 2000iW206921702 | 2021 | - |
| PM2.5 | Thermo Scientific | 2000i | 2000i2 0370 1306 | 2018 | - |
| PM2.5 | Thermo Scientific | 2000i | 2000i2 0433 1312 | 2019 | - |
| PAH | Tisch Environmental | TE-PIF+BL | 1001057 | 2016 | - |
| VOC | Global Analyzer Systems | G23MTS-2CH | 2021-101 | 2023 | - |
| VOC | Global Analyzer Systems | G23MTS-2CH | 2021-102 | 2023 | - |
| Dustfall | Advantage Manufacturing | - | - | 2022 | - |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | G0840069 | 3 | 2020 | January, 1999 |
| WD | Met One | 020C-1 | G3212 | 3 | 2021 | January, 1999 |
| WS | Met One | 010C-1 | R14656 | 3 | 2019 | January, 1999 |
| BP | Young | 61302V-10 | BPA4395 | 3 | 2016 | December, 2012 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 8205 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 3805 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701H | 198 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | N/A |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton |  |

Figure 1 – Area topographic map showing AMS 07

Figure 2 – Aerial image showing AMS 07

Figure 3 – Plan view image for AMS 07 site

Figure 4 – Elevation view image for AMS 07 site

Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5 – Environment looking North

Figure 6 – Environment looking East

Figure 7 – Environment looking South

Figure 8 – Environment looking West

Figure 9 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold

Figure 11 – Curb shot of the monitoring station

Figure 12 –Photo of the front and the back of instrument rack

Figure 13 – Windrose (2021-2025)