Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# AMS 08 – Fort Chipewyan

LAST UPDATED: March 13, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Station Photos	16

Tables and Figures

Figure 1 – Area topographic map showing AMS 08	8

Figure 2 – Aerial photo showing AMS 08	9

Figure 3 – Plan view sketch for AMS 08 site	10

Figure 4 – Environment looking North	11

Figure 5 – Environment looking East	12

Figure 6 – Environment looking South	13

Figure 7 – Environment looking West	14

Figure 8 – Meteorological Tower	15

Figure 9 – Photo showing the inlet and sample manifold	16

Figure 10 – Curb shot of the monitoring station	17

Figure 11 –Photo of the front and the back of instrument rack	18

Figure 12 – Windrose (2021-2025)	19

# General Site Information

Revision Date: March 13, 2026

## Station

| Station ID | AMS 08 |
| --- | --- |
| Station name | Fort Chipewyan |
| Date station established | 1998 |

## Location

| Station street address | Fort Chipewyan |
| --- | --- |
| Legal land description | 6-07-112-07 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 58.709285 |
| Longitude | -111.175014 |
| UTM East | 489862 |
| UTM North | 6507689 |
| Nearest community | Located in Fort Chipewyan |
| Community population | 798 |
| Census Year | 2021 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | NA |
| Approval number | NA |
| Contact Name | Wood Buffalo Environmental Association |
| Address | Unit 3 – 805 Memorial Drive, Fort McMurray, AB |
| Phone number | 780-799-4420 |
| Email address | info@wbea.org |

## Site Description

| Land use by sector | 0 – 90 degrees | Residential |
| --- | --- | --- |
|  | 91 – 180 degrees | Residential, Lake |
|  | 181 – 270 degrees | Residential, Lake |
|  | 271 – 360 degrees | Residential |
| Site elevation (m) (above sea level) | 221 m |  |
| Angle of elevation to nearby buildings | Greatest angle | 0 |
|  | Building direction | NA |
| Airflow restrictions | North | No |
|  | East | No |
|  | South | No |
|  | West | No |
| Distance to nearest trees (m) | North | 50m |
|  | East | NA |
|  | West | NA |
|  | South | NA |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10 |
|  | Distance from station (m) | 0 |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| House | About 20 m, South | Local residential house; potential for wood-burning. |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Driveway | Low | 10 | Dirt road, residential use |
| Local roads | Low | 100 | Paved road, very low volume |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Suncor/Syncrude | Oil Sands Production | 160 | South |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Environmental | 43i-TLE | 1170050147 | 2014 | January, 1999 |
| CO | Teledyne API | T300 | 3505 | 2019 | April, 2019 |
| CO2 | Teledyne API | T360 | 289 | 2020 | April, 2019 |
| NOx/NO/NO2 | Thermo Environmental | 42i | 1426262592 | 2022 | January, 1999 |
| O3 | Thermo Scientific | 49i | 1152220026 | 2021 | January, 1999 |
| TRS | Thermo Environmental | 43i-TLE | 1170050147 | 2022 | - |
| PM 2.5 | Teledyne API | T 640 | 319 | 2018 | February, 2000 |
| Time-Integrated |  |  |  |  |  |
| VOC | Tisch | TE-123 | 1029 | 2023 | - |
| PM10 | Thermo Scientific | 2000i | 2000IW-20929-2106 | 2023 | - |
| PM10 | Thermo Scientific | 2000i | 2000IW-20933-2106 | 2023 | - |
| PM2.5 | Thermo Scientific | 2000i | 2000IW-20932-2106 | 2023 | - |
| PM2.5 EC/OC | Thermo Scientific | 2000i | 2000IW-20934-2106 | 2023 | - |
| PAH | Tisch | TE-PUF+BL | 100111 | 2023 | - |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| PC | OTT | Pluvio 2 – 400 | 358347 | 2 | March, 2023 | February, 1999 |
| AT/RH | Vaisala | HMP155 | F5010004 | 5 | March, 2014 | January, 2008 |
| WS | Met One | 010C-1 | P22394 | 2 | March, 2014 | January, 1999 |
| WD | Met One | 020C-1 | P22884 | 2 | March, 2014 | January, 1999 |
| LW | Campbell Scientific | LWS-L | 23946-3 | NA | March, 2014 | February, 1999 |
| GR | Eppley Lab Inc. | 8-48 | 38243 | 1 | March, 2014 | January, 1999 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 11039 |
| Datalogger | Datalogger | Campbell Scientific | CR310 | 2237 |
| Zero air generator | Zero Air Generator | Teledyne/API | 701 | 197 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | 314B132990230-02 |
| Shelter / Building | Air monitoring portable | ITB | 10 x 20 trailer | 13 15920 |
| Gas Dilution Calibrator | Mass flow-controlled gas dilution | Teledyne/API | T700 | 3810 |
| Hydrogen Generator | Hydrogen Generator | Peak Scientific | 100cc | 720121160 |
| Nitrogen Generator | Nitrogen Generator | Peak Scientific | NG5000A | 771048318 |

Figure 1 – Area topographic map showing AMS 08

Figure 2 – Aerial photo showing AMS 08

Figure 3 – Plan view sketch for AMS 08 site

Site photos

The following photos show the environment surrounding the monitoring station.

Figure 4 – Environment looking North

Figure 5 – Environment looking East

Figure 6 – Environment looking South

Figure 7 – Environment looking West

Figure 8 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 9 – Photo showing the inlet and sample manifold

Figure 10 – Curb shot of the monitoring station

Figure 11 –Photo of the front and the back of instrument rack

Figure 12 – Windrose (2021-2025)