Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Fort McKay South

LAST UPDATED: March 12, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	11

Station Photos	14

Tables and Figures

Figure 1 – Area Topographic map showing AMS 13	7

Figure 2 – Aerial image showing AMS 13	8

Figure 3 – Plan view image for AMS 13 – Fort McKay South	9

Figure 4 – Elevation image for AMS 13 – Fort McKay South	10

Figure 5 – Environment looking North	   Figure 6 – Environment looking East	11

Figure 7 – Environment looking South	   Figure 8 – Environment looking West	12

Figure 9 – Meteorological Tower	13

Figure 10 – Photo showing the inlet and sample manifold.	14

Figure 11 – Curb shot of the monitoring station.	15

Figure 12 –Photo of the front and the back of instrument rack	16

Figure 13 – Windrose (2021-2025)	17

# General Site Information

Revision Date: February 18, 2026

## Station

| Station ID | AMS 13 |
| --- | --- |
| Station name | Fort McKay South |
| Date station established | 2002 |

## Location

| Station street address | None |
| --- | --- |
| Legal land description | 4-13-094-11 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 57.149175 |
| Longitude | -111.642344 |
| UTM East | 461136 |
| UTM North | 6334175 |
| Nearest community | Fort McKay |
| Community population | 757 |
| Census Year | 2021 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | Syncrude Canada Ltd |
| Approval number | 026-03-00 |
| Contact Name | Nikola Stears |
| Address | Bag 4009, MD 4160, Fort McMurray, Alberta, T9H 3L1 |
| Phone number | 780-742-7281 |
| Email address | nstears@suncor.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Forest |
| --- | --- | --- |
|  | 91 – 180 degrees | Forest |
|  | 181 – 270 degrees | Forest |
|  | 271 – 360 degrees | Forest |
| Site elevation (m) (above sea level) | 268 m |  |
| Angle of elevation to nearby buildings | Greatest angle | None |
|  | Building direction | None |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | None |
| Distance to nearest trees (m) | North | 150 |
|  | East | 108 |
|  | West | 56 |
|  | South | 150 |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10 m |
|  | Distance from station (m) | 0 m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| None | n/a | n/a |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Asphalt | High | 326 | Main Road to Fort McKay and CNRL Horizon |
| Dirt | Low | 99 | Road to AMS 13 |
| Dirt | Very Low | 10 | Access Road to AMS 13 |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| CNRL Horizon | OilSands Plant | 21.9 | N |
| Syncrude | Tailings pond | 6.2 | SW |
| Syncrude | OilSands Plant | 12.3 | S |
| Fort McKay Enterprises Ltd. | Laydown Yard | 1 | SE |
| Bouchier Group | LayDown Yard | 1 | SE |
| BME Ltd. | Laydown Yard | 1 | SE |
| Wood Buffalo Water Treatment Plant | Water Treatment Plant | 4.9 | N |
| CNRL Albian | Tailings pond | 9.2 | NE |
| CNRL Albian | Oilsands Plant | 14.1 | NE |

# Station Equipment

Equipment Owner: Syncrude Canada Ltd.

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Teledyne/API | 43i-TLE | 1331259320 | 2025 | January, 1999 |
| TRS | Thermo Scientific | 43i-TLE | 1180540017 | 2016 | January, 1999 |
| NO/NOx/NO2 | Thermo Scientific | 42i | 12300522720 | 2025 | January, 1999 |
| THC/CH4/NMHC | Thermo Scientific | 55i | 1181490018 | 2025 | January, 1999 |
| O3 | Teledyne/API | T400 | 7413 | 2025 | January, 1999 |
| PM2.5 | Teledyne/API | T640 | 1335 | 2023 | January, 1999 |
| Time-Integrated |  |  |  |  |  |
| PM10 | Thermo Scientific | 2000i | 20001203861408 | 2016 | - |
| VOC | Tisch | TE123 | 1023 | 2016 | - |
| TRS converter | Thermo Scientific | CDN-101 | 521 | 2016 |  |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | G4340047 | Class 3 | 2016 | January, 2002 |
| WS | Met One | 010C-1 | N10022 | Class 4 | 2022 | January, 1999 |
| WD | Met One | 020C-1 | W16101 | Class 4 | 2021 | January, 1999 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 11038 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 2448 |
| Zero air generator | Zero Air Generator | Teledyne/API | 701 | 1118 |
| Shelter / Building | Air monitoring portable | C&V | 10 x 20 trailer | 18751-1 |
| HVAC | Heating and air conditioning system. Wall mount unit | Intherem | E2EB-010HB | E2E011101302 |
| TRS Converter | Thermal Oxidizer | C.D.Nova | CDN-101 | 521 |

Figure 1 – Area Topographic map showing AMS 13

Figure 2 – Aerial image showing AMS 13

Figure 3 – Plan view image for AMS 13 – Fort McKay South

Figure 4 – Elevation image for AMS 13 – Fort McKay South

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5 – Environment looking North				Figure 6 – Environment looking East

Figure 7 – Environment looking South				Figure 8 – Environment looking West

Figure 9 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold.

Figure 11 – Curb shot of the monitoring station.

Figure 12 –Photo of the front and the back of instrument rack

Figure 13 – Windrose (2021-2025)