Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Conklin

LAST UPDATED: March 02, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorolgical Equipment	6

Support Equipment	7

Site photos	12

Station Photos	17

Tables and Figures

Figure 1 – Area Topographic map showing AMS 21	8

Figure 2 – Plan view sketch for AMS 21 site	9

Figure 3 – Aerial photo showing AMS 21	10

Figure 5 – Environment Looking North	12

Figure 6 – Environment Looking East	13

Figure 7 – Environment looking South	14

Figure 8 – Environment Looking West	15

Figure 9 – Meteorological Tower	16

Figure 10 – Photo showing the inlet and sample manifold.	17

Figure 11 – Curb shot of the monitoring station.	18

Figure 12 –Photo of front and back of instrument rack	19

Figure 13 – Windrose (2021-2025)	20

# General Site Information

Revision Date: March 28, 2024

## Station

| Station ID | AMS 21 |
| --- | --- |
| Station name | Conklin |
| Date station established | April 01, 2016 |

## Location

| Station street address | Father Mercredis Trail |
| --- | --- |
| Legal land description | 15-31-076-07 W4 |
| Latitude | 55.632330 |
| Longitude | -111.078877 |
| UTM East | 495034 |
| UTM North | 6165163 |
| Elevation | 559.9 |
| Nearest community | Conklin |
| Community population | 185 |
| Census Year | 2016 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Name of Approval Holder | NA |
| Approval number | NA |
| Contact Name | Wood Buffalo Environmental Association |
| Address | Unit 3 – 805 Memorial Drive, Fort McMurray, AB |
| Phone number | 780-799-4420 |
| Email address | info@wbea.org |

## Site Description

| Land use by sector | 0 – 90 degrees | Forest and Residential |
| --- | --- | --- |
|  | 91 – 180 degrees | Forest and Residential |
|  | 181 – 270 degrees | Forest and Residential |
|  | 271 – 360 degrees | Forest and Residential |
| Site elevation (above sea level) | 562 |  |
| Angle of elevation to nearby buildings | Greatest angle | 0 degree |
|  | Building direction | South – CRDAC office |
| Airflow restrictions | North | Trees |
|  | East | None |
|  | South | None |
|  | West | Conklin Rec center, about 2 km west |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 meter |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground | 10 m |
|  | Distance from station | Attached to north site of the station |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Wetlands | 20 | Peat bog / Marshes – Variety of reeds and grasses. |
| Admin building | 20 | Conklin Resource Development Advisory Committee Office |
| Shipping Container | 20 | SEA-CAN |
| Free-Standing Structure | 20 | Gazebo – made of wood. |
| Watercraft | 20 | Speed Boat |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Gravel | Medium | 50 | Father Mercredis Trail – Access Road |
| Paved | Medium | 200 | Northland Drive |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Meg Energy | Oil and Gas | 25 | NE |
| Cenovus Energy | Oil and Gas | 14 | SE |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Environmental | 43i | 1428701363 | 2016 | April, 2016 |
| TRS | Thermo Environmental | 43i-LTE | 12228021058 | 2024 | April, 2016 |
| TRS | CD Nova | CDN-101 | 565 | 2025 |  |
| NO/NOx/NO2 | Thermo Environmental | 42i | 1218153356 | 2025 | April, 2016 |
| THC/CH4/NMHC | Thermo Environmental | 55i | 1180320039 | 2024 | April, 2016 |
| O3 | Thermo Environmental | 49i | 1300156233 | 2025 | April, 2016 |
| PM2.5 | Teledyne/API | T640 | 1266 | 2025 | April, 2016 |
| Time-Integrated |  |  |  |  |  |
| PM2.5 | Thermo Environmental | 2000i | 2000iW208842002 | 2020 | - |
| PM2.5 | Thermo Environmental | 2000i | 2000iW208822002 | 2020 | - |
| PM10 | Thermo Environmental | 2000i | 2000iW208812022 | 2020 | - |
| PM10 | Thermo Environmental | 2000i | 2000iW208832002 | 2020 | - |
| VOC | Tisch Environmental | TE-123 | 1019 | 2020 | - |
| PAH | Tisch Environmental | TE-PUF | 1001100 | 2020 | - |
| Dustfall | Advantage Manufacturing | - | - | 2022 | - |

## Meteorolgical Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | G4330044 | 3 | 2025 | April, 2016 |
| WD | Met One | 010C-1 | G3858 | 2 | 2025 | April, 2016 |
| WS | Met One | 020C-1 | J433 | 2 | 2016 | April, 2016 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 9628 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 2656 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701H | 355 |
| Shelter / Building | Air monitoring portable | ITB | 10x20 trailer | ITB-14-16423 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | NA |
| Tower | 10 Tower | Aluma | T-135 | AT-215036-AA-5-3 |
| Deck | Wooden Deck | NA | 10x20 | NA |

Figure 1 – Area Topographic map showing AMS 21

Figure 2 – Plan view sketch for AMS 21 site

Figure 3 – Aerial photo showing AMS 21

Figure 4 – Cross Section Elevation Drawing of AMS 21

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5 – Environment Looking North

Figure 6 – Environment Looking East

Figure 7 – Environment looking South

Figure 8 – Environment Looking West

Figure 9 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold.

Figure 11 – Curb shot of the monitoring station.

Figure 12 –Photo of front and back of instrument rack

Figure 13 – Windrose (2021-2025)