Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Janvier

LAST UPDATED: February 19, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	12

Station Photos	17

Tables and Figures

Figure 1 – Area topographic map showing AMS 22	8

Figure 2 – Aerial photo showing AMS 22	9

Figure 3 – Plan view sketch for AMS 22 site	10

Figure 4 – Elevation view image for AMS 22 site	11

Figure 5 – Environment looking North	12

Figure 6 – Environment looking East	13

Figure 7 – Environment looking South	14

Figure 8 – Environment looking West	15

Figure 9 – Meteorological Tower	16

Figure 10 – Photo showing the inlet and sample manifold	17

Figure 11 – Curb shot of the monitoring station	18

Figure 12 –Photo of the front and the back of instrument rack	20

Figure 13 – Windrose (five year)	21

# General Site Information

Revision Date: March 28, 2024

## Station

| Station ID | AMS 22 |
| --- | --- |
| Station name | Janvier |
| Date station established | October 2016 |

## Location

| Station street address | Block 4; Lot 135 - Adjacent to Nokohoo Road between Teed Ave and Lapouse Ave. |
| --- | --- |
| Legal land description | 6-05-80-05-W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 55.903242 |
| Longitude | -110.749758 |
| UTM East | 515647 |
| UTM North | 6195323 |
| Nearest community | Janvier |
| Community population | 437 |
| Census Year | 2021 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | NA |
| Approval number | NA |
| Contact Name | Wood Buffalo Environmental Association |
| Address | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Phone number | 780-799-4420 |
| Email address | info@wbea.org |

## Site Description

| Land use by sector | 0 – 90 degrees | Trees |
| --- | --- | --- |
|  | 91 – 180 degrees | Main road |
|  | 181 – 270 degrees | Main road |
|  | 271 – 360 degrees | Trees and House |
| Site elevation (m) (above sea level) | 471 |  |
| Angle of elevation to nearby buildings | Greatest angle | 10° |
|  | Building direction | SE |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | Trees (45 m from station, 17 m high) |
| Distance to nearest trees (m) | North | 80 m from station, 17 m high |
|  | East | 80 m from station, 15 m high |
|  | West | 45 m from station, 17 m high |
|  | South | None |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 20 m |
|  | Distance from station (m) | 2 m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Vehicles | 20 m west | Gravel road |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Main access road | low | 25 m | Paved |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| ConocoPhillips | SAGD | 33.38 | NW |
| Tervita | Landfill | 11 kms | NW |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Scientific | 43i | 1152430006 | 2016 | January, 2017 |
| TRS | Thermo Scientific | 43i-TLE | 1200326169 | 2023 | January, 2017 |
| NO/NOx/NO2 | Thermo Scientific | 42i | 1229254994 | 2025 | January, 2017 |
| THC/CH4/NMHC | Thermo Scientific | 55i | 1317958219 | 2024 | January, 2017 |
| O3 | Thermo Scientific | 49i | 1227254861 | 2025 | January, 2017 |
| PM 2.5 | Teledyne/API | T640 | 325 | 2018 | January, 2017 |
| Time-Integrated |  |  |  |  |  |
| PM 2.5 A | Thermo Scientific | 2000i | 2000i 20388 1308 | 2019 | - |
| PM 2.5 B | Thermo Scientific | 2000i | 2000i 20489 1408 | 2019 | - |
| PM10 A | Thermo Scientific | 2000i | 2000i 20523 1411 | 2020 | - |
| PM10 B | Thermo Scientific | 2000i | 2000i 20383 1308 | 2020 | - |
| PAH | Tisch | TE-PUFPLUSBL | 1001108 | 2021 | - |
| VOC | Tisch | TE-123 | 1018 | 2021 | - |
| Dustfall | Advantage Manufacturing | - | - | 2022 | - |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | G4330042 | Class 3 | 2018 | January, 2016 |
| WS 20m | Met One | 010C-1 | J4337 | Class 3 | 2025 | October, 2021 |
| WD 20m | Met One | 020C-1 | Y5194 | Class 3 | 2025 | October, 2021 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 2586 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 3806 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701 | 691 |
| TRS converter | Thermal oxidizer | CD Nova | CDN-101 | 620 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | ITB-15-16494 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | 314P143189505-02 |
| Deck | 10x20 | NA | NA | NA |

Figure 1 – Area topographic map showing AMS 22

Figure 2 – Aerial photo showing AMS 22

Figure 3 – Plan view sketch for AMS 22 site

Figure 4 – Elevation view image for AMS 22 site

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5 – Environment looking North

Figure 6 – Environment looking East

Figure 7 – Environment looking South

Figure 8 – Environment looking West

Figure 9 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold

Figure 11 – Curb shot of the monitoring station

Figure 12 -Photo of the front and the back of the instrument rack

Figure 13 –Photo of the front and the back of instrument rack

Figure 14 – Windrose (five year)