Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Fort Hills

LAST UPDATED: March 02,2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	9

Station Photos	14

Tables and Figures

Figure 1 – Aerial photo showing AMS 23	7

Figure 2 – Plan view sketch for AMS 23 site	8

Figure 3 – Environment looking North	9

Figure 4 – Environment looking East	10

Figure 5 – Environment looking South	11

Figure 6 – Environment looking West	12

Figure 7 – Meteorological Tower	13

Figure 8 – Photo showing the inlet and sample manifold	14

Figure 9 – Curb shot of the monitoring station	15

Figure 10 –Photo of the front and the back of instrument rack	16

Figure 11 –Photo of the T640 PM monitor and inlet	17

Figure 12 – Windrose (2020-2025)	18

# General Site Information

Revision Date: March 28, 2024

## Station

| Station ID | AMS 23 |
| --- | --- |
| Station name | Fort Hills |
| Date station established | March 2017 |

## Location

| Station street address | Located North of the Southwest Raw Water Pond |
| --- | --- |
| Legal land description | 15-24-096-11 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 57.3489012253 |
| Longitude | -111.639688536 |
| UTM East | 461505 |
| UTM North | 6356406 |
| Nearest community | Fort McKay |
| Community population | 60 |
| Census Year | 2025 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3 - 805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | Fort Hills Energy Corporation |
| Approval number | 151469-01-00 |
| Contact Name | Ty Callioux |
| Address | P.O. Box 4001 Fort McMurray, Alberta, T9H 3E3 |
| Phone number | 780-743-7780 (business hours); 780-762-4200 (non-business hours) |
| Email address | tcallioux@suncor.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Water treatment building |
| --- | --- | --- |
|  | 91 – 180 degrees | Raw Water Pond |
|  | 181 – 270 degrees | Raw Water Pond |
|  | 271 – 360 degrees | Access road, drilling lay down yard |
| Site elevation (m) (above sea level) | 283m |  |
| Angle of elevation to nearby buildings | Greatest angle | 20° |
|  | Building direction | West |
| Airflow restrictions | North | N/A |
|  | East | N/A |
|  | South | N/A |
|  | West | N/A |
| Distance to nearest trees (m) | North | N/A |
|  | East | N/A |
|  | West | N/A |
|  | South | N/A |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10m |
|  | Distance from station (m) | 0m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Raw water pond | 20m | Storage pond for raw process water |
| Trailer Laydown | 20m | Storage for Trailers |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Gravel Road | Medium | 100 | Mine access road used by pickup trucks and heavy equipment |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Fort Hills | Oil Sands Plant | 0 | N/A |
| CNRL Horizon | Oil Sands Plant | 10 | Southwest |
| CNRL Albian | Oil Sands Plant | 15 | South |

# Station Equipment

Equipment Owner: Suncor – Fort Hills

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Scientific | 43i | 1160290012 | December, 2021 | June, 2017 |
| TRS | Thermo Scientific | 43i-TLE | 1300156232 | December, 2021 | June, 2017 |
| NO/NOx/NO2 | Thermo Scientific | 42i | 1152430007 | December, 2021 | June, 2017 |
| THC/CH4/NMHC | Thermo Scientific | 55i | 12227620777 | December, 2025 | July, 2017 |
| PM2.5 | Teledyne API | T640 | 320 | December, 2025 | June, 2017 |
| Time-Integrated |  |  |  |  |  |
| PM10 A | Thermo Scientific | 2000i-A-N | 2000I2-0382-308 | December, 2021 | - |
| PM10 B | Thermo Scientific | 2000i-A-N | 2000IW 20930 2108 | December, 2021 | - |
| VOC | Global Analyzer Systems | G170 | 2023-234 | 2023 | - |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | T2950501 | Class 3 | December, 2021 | June, 2017 |
| WS | Met One | 010C-1 | E1038 | Class 3 | December, 2025 | June, 2017 |
| WD | Met One | 020C-1 | B17268 | Class 3 | December, 2025 | June, 2017 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 7882 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 451 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701 | 1117 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 14 trailer | 2C9CSC2G7M1044018 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | 12345 |
| TRS Converter | Thermal Oxidizer | CD Nova | CDN-101 | 594 |
| H2Generator | Hydrogen Generator | Parker Hannifin | 63-0100 | 14950646 |

Figure 1 – Aerial photo showing AMS 23

Figure 2 – Plan view sketch for AMS 23 site

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 3 – Environment looking North

Figure 4 – Environment looking East

Figure 5 – Environment looking South

Figure 6 – Environment looking West

Figure 7 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 8 – Photo showing the inlet and sample manifold

Figure 9 – Curb shot of the monitoring station

Figure 10 –Photo of the front and the back of instrument rack

Figure 11 –Photo of the T640 PM monitor and inlet

Figure 12 – Windrose (2020-2025)