Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Ells River

LAST UPDATED: March 02, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorolgical Equipment	6

Support Equipment	6

Site photos	11

Station Photos	16

Tables and Figures

Figure 1 – Area Topographic map showing AMS 30	7

Figure 3 – Aerial photo showing AMS 30	9

Figure 5 – Environ Looking North	11

Figure 6 – Environ Looking East	12

Figure 7 – Environ looking South.	13

Figure 8 – Environ Looking West	14

Figure 9 – Meteorological Tower	15

Figure 10 – Photo showing the inlet and sample manifold	16

Figure 11 – Curb shot of the monitoring station	17

Figure 12 –Photo of front and back of instrument rack	18

Figure 13 – Windrose (2021-2025)	19

# General Site Information

Revision Date: February 18, 2026

## Station

| Station ID | AMS 30 |
| --- | --- |
| Station name | Ells River |
| Date station established | 2020 |

## Location

| Station street address | Located at about 300 m northwest of the Total Joslyn Camp. |
| --- | --- |
| Legal land description | 12-04-096-11 W4 |
| Latitude | 57.241180 |
| Longitude | -111.722081 |
| Ground Elevation | 299.9 |
| UTM East | 456424 |
| UTM North | 6344478 |
| Nearest community | Fort Mackay |
| Community population | 621 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Name of Approval Holder | Canadian Natural Resources Ltd. |
| Approval number | 149968-01-00 |
| Contact Name | Malathi Velmurugan |
| Address | 2100, 855 – 2 Street SW Calgary, AB T2P 4J8 |
| Phone number | (780) 714-4436 |
| Email address | Malathi.Velmurugan@cnrl.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Pond |
| --- | --- | --- |
|  | 91 – 180 degrees | Forest / Highway |
|  | 181 – 270 degrees | Forest |
|  | 271 – 360 degrees | Forest |
| Site elevation (above sea level) | 304 |  |
| Angle of elevation to nearby buildings | Greatest angle | 12 degrees |
|  | Building direction | East |
| Airflow restrictions | North | No |
|  | East | No |
|  | South | No |
|  | West | No |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground | 10 metres |
|  | Distance from station | 7 metres |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Cell tower compound | 14 | Compound that holds the equipment for the cell tower to function |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Paved road | Very low | 50 M West | Old horizon highway |
| Paved road | High | 250 M East | Horizon highway |
| Dirt road | Very low | 100 M East | Dirt road around the pond |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| CNRL Horizon Oil Sands | Oil plant | 10km | North |
| CNRL Horizon | Open mining | 7km | North |

# Station Equipment

Equipment Owner: Canadian Natural Resources Ltd.

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| Continuous |  |  |  |  |  |
| SO2 | Thermo Scientific | 43i | 1008841397 | 2021 | October, 2020 |
| TRS | Thermo Scientific | 43i-TLE | 1170050152 | 2021 | October, 2020 |
| NO/NOx/NO2 | Thermo Scientific | 42i | 710321429 | 2021 | October, 2020 |
| THC/CH4/NMHC | Thermo Scientific | 55i | 1152430011 | 2023 | October, 2020 |
| PM 2.5 | Teledyne/API | T640 | 875 | 2021 | October, 2020 |
| Time-Integrated |  |  |  |  |  |
| PM10 A | Thermo Environmental | 2000i | 2000I2 204961409 | 2021 | - |
| PM10 B | Thermo Environmental | 2000i | 2000IW 206011510 | 2021 | - |
| TSP | Thermo Environmental | 2000i | 2000IW 209632110 | 2023 | - |
| VOC | Tisch Environmental | Te-123 | 1030 | 2021 | - |

## Meteorolgical Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | F5010003 | 3 | 2021 | October, 2020 |
| WS | Met One | 010C-1 | A3111 | 5 | 2021 | October, 2020 |
| WD | Met One | 020C-1 | E5131 | 5 | 2021 | October, 2020 |
| GR | Eppley Lab Inc. | 8-48 | 34276 | n/a | 2021 | October, 2020 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 11040 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 5707 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701H | 358 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | 2N9MF73895 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | NA |
| TRS Converter | Thermal oxidizer | CD Nova | CDN-101 | 562 |

Figure 1 – Area Topographic map showing AMS 30

Figure 2 – Plan view sketch for AMS 30 site

Figure 3 – Aerial photo showing AMS 30

Figure 4 – Cross-Sectional Elevation Drawing of AMS 30

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5 – Environ Looking North

Figure 6 – Environ Looking East

Figure 7 – Environ looking South.

Figure 8 – Environ Looking West

Figure 9 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold

Figure 11 – Curb shot of the monitoring station

Figure 12 –Photo of front and back of instrument rack

Figure 13 – Windrose (2021-2025)