Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Leismer

LAST UPDATED: February 18, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	10

Station Photos	13

Tables and Figures

Figure 1 – Area Topographic map showing AMS 501	7

Figure 2– Aerial photo showing AMS 501	8

Figure 3 – Plan view sketch for AMS 501 - Leismer	9

Figure 4 – Environment looking North	   Figure 5 – Environment looking East	10

Figure 6 – Environment looking South	   Figure 7 – Environment looking West	11

Figure 8 – Meteorological Tower	12

Figure 9 – Photo showing the inlet and sample manifold	13

Figure 10 – Curb shot of the monitoring station	14

Figure 11 –Photo of the front and the back of instrument rack	15

Figure 12 – Windrose (2021-2025)	16

# General Site Information

Revision Date: February 18, 2026

## Station

| Station ID | AMS 501 |
| --- | --- |
| Station name | Leismer |
| Date station established | June 2013 |

## Location

| Station street address | Located at the main gate of the Athabasca Oil Corporation SAGD plant. |
| --- | --- |
| Legal land description | 4-01-079-10 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 55.81294255 |
| Longitude | -111.44054535 |
| UTM East | 472392 |
| UTM North | 6185349 |
| Nearest community | Conklin |
| Community population | 154 |
| Census Year | 2021 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | Athabasca Oil Corporation |
| Approval number | 289664-01-00; 241311-01-00 |
| Contact Name | Peter Millman |
| Address | NA |
| Phone number | 587-233-1382 |
| Email address | pmillman@atha.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Gate/Main Road |
| --- | --- | --- |
|  | 91 – 180 degrees | Road/Trees |
|  | 181 – 270 degrees | Trees |
|  | 271 – 360 degrees | Trees/laydown yard |
| Site elevation (m) (above sea level) | 668 m |  |
| Angle of elevation to nearby buildings | Greatest angle | 0 |
|  | Building direction | NA |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | None |
| Distance to nearest trees (m) | North | 150 |
|  | East | 150 |
|  | West | 35 |
|  | South | 35 |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10 m |
|  | Distance from station (m) | 0 m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| SAGD operations | 100 m | Athabasca Oil SAGD operations |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Roadway | Medium | 15 m | Main road access into SAGD plant |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Atha Oil corp. | SAGD | 100 m | West |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| SO2 | Thermo Scientific | 43i | 1160290011 | 2018 | July, 2014 |
| H2S | Thermo Scientific | 43i-TLE | 1180540020 | 2023 | July, 2014 |
| NO/NOx/NO2 | Thermo Scientific | 42iQ | 12400232071 | 2025 | July, 2014 |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | N5050067 | Class 3 | 2025 | July, 2014 |
| WS | Met One | 010C-1 | Y18362 | Class 4 | 2023 | July, 2014 |
| WD | Met One | 020C-1 | R14654 | Class 4 | 2023 | July, 2014 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 11041 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 3252 |
| Zero air generator | Zero Air Generator | Teledyne/API | 701 | 4427 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | ITB1315940 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | NA |

Figure 1 – Area Topographic map showing AMS 501

Figure 2– Aerial photo showing AMS 501

Figure 3 – Plan view sketch for AMS 501 - Leismer

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 4 – Environment looking North				Figure 5 – Environment looking East

Figure 6 – Environment looking South				Figure 7 – Environment looking West

Figure 8 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 9 – Photo showing the inlet and sample manifold

Figure 10 – Curb shot of the monitoring station

Figure 11 –Photo of the front and the back of instrument rack

Figure 12 – Windrose (2021-2025)