Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Sawbones Bay

LAST UPDATED: February 18, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	10

Station Photos	13

Tables and Figures

Figure 1 – Area Topographic map showing AMS 505	7

Figure 2 – Aerial image showing AMS 505	8

Figure 3 – Plan view sketch for AMS 505 – Sawbones Bay	9

Figure 4 – Environment looking North.	  Figure 5 – Environment looking East.	10

Figure 6 – Environment looking South.	  Figure 7 – Environment looking West.	11

Figure 8 – Meteorological Tower	12

Figure 9 – Photo showing the inlet and sample manifold.	13

Figure 10 – Curb shot of the monitoring station.	14

Figure 11 –Photo of the front and the back of instrument rack	15

Figure 12 – Windrose (2021-2025)	16

# General Site Information

Revision Date: February 18, 2026

## Station

| Station ID | AMS 505 |
| --- | --- |
| Station name | Sawbones Bay |
| Date station established | June 23, 2021 |

## Location

| Station street address | Station located in laydown yard at Christina Lake facility |
| --- | --- |
| Legal land description | 1-16-77-5 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 55.667847 |
| Longitude | -110.705711 |
| UTM East | 518511 |
| UTM North | 6169152 |
| Nearest community | Conklin |
| Community population | 229 |
| Census Year | 2018 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | MEG Energy Corp. |
| Approval number | 00216466-01-00 |
| Contact Name | Natasha Rowden |
| Address | NA |
| Phone number | 403-770-5558 |
| Email address | Natasha.rowden@megenergy.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Forest |
| --- | --- | --- |
|  | 91 – 180 degrees | Forest |
|  | 181 – 270 degrees | SAGD operations |
|  | 271 – 360 degrees | Forest |
| Site elevation (m) (above sea level) | 471 m |  |
| Angle of elevation to nearby buildings | Greatest angle | 0 |
|  | Building direction | None |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | None |
| Distance to nearest trees (m) | North | 190 |
|  | East | 75 |
|  | West | NA |
|  | South | 15 |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10 m |
|  | Distance from station (m) | 0 m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| SAGD Operations | 500m W | MEG Energy operations |
| Laydown yard | 100m S | Heavy equipment |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Roadway | Low | 100 | Gravel access road |
| Roadway | Low | 500 | Gravel access road |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| MEG Christina Lake | SAGD operations | 500m | West |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| SO2 | Thermo Scientific | 43i | 710321323 | 2021 | July, 2017 |
| H2S | Thermo Scientific | 43i-QTL | 12113311965 | 2023 | July, 2017 |
| NO/NOx/NO2 | Teledyne/API | T200 | 4259 | 2025 | July, 2017 |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | N2910504 | Class 3 | 2021 | July, 2017 |
| WS | Met One | 010C-1 | CA 03845 | Class 4 | 2023 | July, 2017 |
| WD | Met One | 020C-1 | B4693 | Class 4 | 2023 | July, 2017 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 6894 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 3807 |
| Zero air generator | Zero Air Generator | Teledyne/API | 701 | 690 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | ITB1315940 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton | NA |
| H2S Converter | H2S Converter | Direct Optics | G150 | 2022-224 |

Figure 1 – Area Topographic map showing AMS 505

Figure 2 – Aerial image showing AMS 505

Figure 3 – Plan view sketch for AMS 505 – Sawbones Bay

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 4 – Environment looking North.			Figure 5 – Environment looking East.

Figure 6 – Environment looking South.			Figure 7 – Environment looking West.

Figure 8 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 9 – Photo showing the inlet and sample manifold.

Figure 10 – Curb shot of the monitoring station.

Figure 11 –Photo of the front and the back of instrument rack

Figure 12 – Windrose (2021-2025)