Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# Kirby North

LAST UPDATED: March 2, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	10

Station Photos	15

Tables and Figures

Figure 1 – Area topographic map showing AMS 508	7

Figure 2 – Plan view sketch for AMS 508 site	8

Figure 3 – Aerial photo showing AMS 508	9

Figure 4 – Environment looking North	10

Figure 5 – Environment looking East	11

Figure 6 – Environment looking South	12

Figure 7 – Environment looking West	13

Figure 8 – Meteorological Tower	14

Figure 9 – Photo showing the inlet and sample manifold	15

Figure 10 – Curb shot of the monitoring station	16

Figure 11 –Photo of the front and the back of instrument rack	17

Figure 12 – Wind Rose (2021 - 2025)	18

# General Site Information

Revision Date: March 28, 2024

## Station

| Station ID | AMS 508 |
| --- | --- |
| Station name | Kirby North |
| Date station established | 2022 |

## Location

| Station street address | Industrial Pad |
| --- | --- |
| Legal land description | Industrial - SAGD |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 55.4616480 |
| Longitude | -111.2188230 |
| UTM East | 486163 |
| UTM North | 6146187 |
| Nearest community | Conklin |
| Community population | 229 |
| Census Year | 2025 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3-805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | Canadian Natural Resources Ltd. |
| Approval number | 149968-01-00 |
| Contact Name | Shawn Milligan |
| Address | 2100, 855 – 2 Street S.W. Calgary, AB |
| Phone number | (403) 896-3109 |
| Email address | shawn.milligan@cnrl.com |

## Site Description

| Land use by sector | 0 – 90 degrees | SAGD Plant |
| --- | --- | --- |
|  | 91 – 180 degrees | SAGD Land / Forest |
|  | 181 – 270 degrees | SAGD Land / Forest |
|  | 271 – 360 degrees | SAGD Plant |
| Site elevation (m) (above sea level) | 700m |  |
| Angle of elevation to nearby buildings | Greatest angle | None |
|  | Building direction | None |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | None |
| Distance to nearest trees (m) | North | N/A |
|  | East | N/A |
|  | West | N/A |
|  | South | N/A |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 metre |
| Wind Sensors | Type | Cup and vane |
|  | Height above ground (m) | 10m |
|  | Distance from station (m) | Tower is attached to the shelter |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| SAGD Pad | 50 | Station is located on the SE side of the pad |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Dirt Road | Medium | 50 | Road used to access the SAGD plant |

## Major Point Sources

| Facility Name | Source Type | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- |
| Kirby North | SAGD Plant | 100 | W |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Instrument Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- |
| SO2 | Thermo Environmental | 43-iQ | 1182340007 | 2021 | May, 2019 |
| H2S | Thermo Environmental | 43i-TLE | 1150840012 | 2023 | May, 2019 |
| NO/NOx/NO2 | Thermo Environmental | 42-iQ | 1118148496 | 2024 | May, 2019 |
| THC | Thermo Environmental | 51i | 1182340005 | 2021 | May, 2019 |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Sensor Installed | WBEA Data Start Date |
| --- | --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | R3710976 | 3 | 2024 | May, 2019 |
| WS | Met One | 010C-1 | X16479 | 3 | 2021 | May, 2019 |
| WD | Met One | 020C-1 | D13601 | 3 | 2025 | May, 2019 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 2372 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 2445 |
| Zero air generator | Zero Air Generator | Teledyne/API | 701H | 880 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | 17541-1 |
| Tower | 10 metre crank up | Aluma | T-135 | 218148.00.7 |
| H2S converter | H2S converter | Global | G-150 | 2022-197 |

Figure 1 – Area topographic map showing AMS 508

Figure 2 – Plan view sketch for AMS 508 site

Figure 3 – Aerial photo showing AMS 508

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 4 – Environment looking North

Figure 5 – Environment looking East

Figure 6 – Environment looking South

Figure 7 – Environment looking West

Figure 8 – Meteorological Tower

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 9 – Photo showing the inlet and sample manifold

Figure 10 – Curb shot of the monitoring station

Figure 11 –Photo of the front and the back of instrument rack

Figure 12 – Wind Rose (2021 - 2025)