Wood Buffalo Environmental Association

Ambient Air Monitoring Station

Site Documentation

# AMS 14 - Anzac

LAST UPDATED: March 10, 2026

Table of Contents

General Site Information	4

Station	4

Location	4

Owner/Operator/Approval Holder	4

Site Description	4

Site Influences	5

Localized Sources (within 20 metres of station)	5

Roadway Influences	5

Major Point Sources	5

Station Equipment	6

Analytical Equipment	6

Meteorological Equipment	6

Support Equipment	6

Site photos	11

Station Photos	16

Tables and Figures

Figure 1 – Area topographic map showing AMS 14	7

Figure 2 – Aerial photo showing AMS 14	8

Figure 3 – Plan view sketch for AMS 14 site	9

Figure 4 – Elevation view sketch for AMS 14 site	10

Figure 5 – Environment looking North	11

Figure 6 – Environment looking East	12

Figure 7 – Environment looking South	13

Figure 8 – Environment looking West	14

Figure 9 – Meteorological Tower	15

Figure 10 – Photo showing the inlet and sample manifold	16

Figure 11 – Curb shot of the monitoring station	17

Figure 12 –Photo of the front and the back of instrument rack	18

Figure 13 – Windrose 20m (2021-2025)	19

# General Site Information

## Station

| Station ID | AMS 14 |
| --- | --- |
| Station name | Anzac |
| Date station established | January 1, 2006 |

## Location

| Station street address | Stony Mountain Road |
| --- | --- |
| Legal land description | 16-09-086-07 W4 |
| Airshed Zone | Wood Buffalo Environmental Association |
| Latitude | 56.4489083333 |
| Longitude | -111.037975 |
| UTM East | 497659 |
| UTM North | 6256044 |
| Nearest community | Anzac |
| Community population | 659 |
| Census Year | 2025 |

## Owner/Operator/Approval Holder

| Operating Agency | Wood Buffalo Environmental Association |
| --- | --- |
| Address of Operating Agency | Unit 3, 805 Memorial Drive, Fort McMurray, Alberta T9K 0K4 |
| Name of Approval Holder | CNOOC International |
| Approval number | 137467-01-00; 236394-00-00 |
| Contact Name | Nandakumar Rajendran |
| Address | Suite 2300, 500 Centre St. SE, Calgary, AB, T2G 1A6 |
| Phone number | 780-742-6873 |
| Email address | Nandakumar.Rajendran@intl.cnoocltd.com |

## Site Description

| Land use by sector | 0 – 90 degrees | Trees |
| --- | --- | --- |
|  | 91 – 180 degrees | Trees |
|  | 181 – 270 degrees | House |
|  | 271 – 360 degrees | Trailer |
| Site elevation (m) (above sea level) | 497 m |  |
| Angle of elevation to nearby buildings | Greatest angle | 18 |
|  | Building direction | West |
| Airflow restrictions | North | None |
|  | East | None |
|  | South | None |
|  | West | House |
| Sample manifold | Type | All glass |
|  | Inlet height above roof | 1 meter |
| Distance to nearest trees (m) | North | 39 meters |
|  | East | 30 meters |
|  | West | NA |
|  | South | NA |
| Meteorological Sensors | Type | Cup and vane |
|  | Height above ground (m) | 20 m |
|  | Distance from station (m) | 3 m |

# Site Influences

## Localized Sources (within 20 metres of station)

| Type | Distance (m) | Description |
| --- | --- | --- |
| Trailer | 18 m | Communication shack |
| House | 14 m | House W of station |

## Roadway Influences

| Type | Traffic Volume | Distance (m) | Description |
| --- | --- | --- | --- |
| Pavement | Low | 62 m | Main road of Anzac |
| Access road | Low | 16 m | Access road to the station |
| Railway | Low | 70 m |  |

## Major Point Sources

| Facility Name | Source Type | Production Capacity | Distance from site (km) | Compass direction from site |
| --- | --- | --- | --- | --- |
| Nexen Long Lake | SAGD | 70,000 bbl/day | 4.38 | SW |

# Station Equipment

Equipment Owner: WBEA

## Analytical Equipment

| Parameter | Make | Model | Serial Number | Date Installed |
| --- | --- | --- | --- | --- |
| SO2 | Thermo Scientific | 43i | 710321322 | 2023-07-07 |
| TRS | Thermo Scientific | 43i-LTE | 1218153582 | 2022 |
| NO2 | Thermo Scientific | 42i | 1152430008 | 2022 |
| NMHC | Thermo Scientific | 55i | 1331259521 | 2025-01-06 |
| O3 | Thermo Scientific | 49i | 1426262595 | 2024-01-04 |
| PM2.5 | Teledyne | T640 | 825 | 2022 |
| PM10 | Thermo Scientific | 2000i | 2000i 294821408 | 2016 |
| PM10 | Thermo Scientific | 2000i | 2000i 204581405 | 2016 |
| PM2.5 | Thermo Scientific | 2000i | 2000i 203871308 | 2016 |
| PM2.5 | Thermo Scientific | 2000i | 2000iW205911510 | 2016 |
| PAH | Tisch Environmental | TE-PIF+BL | 1001055 | 2016 |
| VOC | Tisch Environmental | TE-123 | 1024 | 2022 |

## Meteorological Equipment

| Parameter | Make | Model | Serial Number | WMO Site Class | Date Installed |
| --- | --- | --- | --- | --- | --- |
| AT/RH | Vaisala | HMP155 | U2120856 | 2 | 2024-11-04 |
| WS | Met One | 010C-1 | U11126 | 3 | 2024-04-09 |
| WD | Met One | 020C-1 | C21020 | 3 | 2024-04-09 |
| Leaf Wetness | Decagon Devices | LWS-L | 23946-6 |  | 2022 |

## Support Equipment

| Name | Description | Make | Model | Serial Number |
| --- | --- | --- | --- | --- |
| Datalogger | Datalogger | Campbell Scientific | CR3000 | 2582 |
| Gas Dilution Calibrator | Dynamic dilution calibrator | Teledyne/API | T700 | 3060 |
| Zero air generator | Zero Air Generator | Teledyne/API | T701H | 357 |
| Shelter / Building | Air monitoring portable | ITB | 8 x 16 trailer | ITB 21 18397 |
| HVAC | Heating and air conditioning system. Wall mount unit | BARD | 1 ton |  |

Figure 1.0 – Area topographic map showing AMS 14.

Figure 2.0 – Aerial image showing AMS 14.

Figure 3.0 – Plan view image for AMS 14 site.

Figure 4.0 – Elevation view image for AMS 14 site.

# Site photos

The following photos show the environment surrounding the monitoring station.

Figure 5.0 – Environment looking North.

Figure 6 – Environment looking East.

Figure 7 – Environment looking South.

Figure 8 – Environment looking West.

Figure 9 – Meteorological Tower.

# Station Photos

The following photos show the monitoring station and instrumentation.

Figure 10 – Photo showing the inlet and sample manifold.

Figure 11 – Curb shot of the monitoring station.

Figure 12 –Photo of the front and the back of instrument rack.

Figure 13 Windrose (2021-2025).